8259 – Programmable Interrupt Controller

8259 is a interrupt priority management device.

The control registers of 8259 are divided into 2 groups.

- Initialization Command Words and

- Operation Command Words

The Initialization Command Words are normally set by an initialization routine when the computer system is booted up and remains constant throughout its operation. The Operation Command words are used to dynamically  control the processing of interrupts.

The few characteristics determined by ICW’S are

- Determines whether the edge triggered mode or level trigerred mode is to be used

- Indicates whether or not the 8259 is cascaded with other 8259 ‘s

- Whether 8259 is in 8086/8088 system or 8085/8088 sytem

The IRR(Interrupt Request Register) ,priority resolver and ISR (Interrupt Service Register )are for receiving and controlling the interrupts that arrive at the IR7-IR0 pins. The IRR latches the incoming requests and in conjunction with the priority resolver , allows the unmasked interrupts( masked interrupts will be given in OCW1 or IMR(Interrupt Mask Register) ) with sufficient priority (which will be resolved with OCW2, which will give the priority of each interrupt ) to put a 1 on the INT pin which will be connected to INTR pin of the 8086 processor.

The prority resolver  logic determines the priorities of the requests in the IRR and ISR is for holding the requests currently being processed.

The OCW1 is also called Interrupt Mask Register(IMR).

After a bit in the IRR is set to 1 it is compared with the corresponding mask bit in the IMR.  If the mask bit is 0, the request is passed on to the priority resolver, but if it is 1, the request is blocked. When an interrupt request is input to the priority resolver , its prority is examined in conjuction with OCW2 and if according to the current state of the priority resolver the interrupt is to be sent to the CPU, the INT line is activated.

The OCW2 is used to set the priority levels of each interrupt.

Assuming that the Interrupt Flag (IF) in the CPU is set to 1 ,the CPU will enter its interrupt sequence at the completion of the current instruction and return two negative pulses over the INTA line. Upon the arrival of the first pulse ,the IRR latches are disabled so that IRR will ignore further signals on the IR7-IR0 lines . This state is maintained until the end of the  second INTA pulse. Also the first INTA pulse will cause the appropriate ISR bit to be set and the corresponding IRR bit to be cleared.

Ordinarily a request on IR0 has the highest priority, one on IR1 has the next highest priority and so on. Under the normal priority mode, if ISRn is set, the priority resolver will not recognize any requests on IR7 through IR(n+1) ,but will recognize unmasked requests on IR(n-1) through IR0.  Consequently if the IF flag in the CPU has been reset to 1, requests having higher  priority than  the one being processed may cause the current interrupt routine to be interrupted with those having lower priorities are kept waiting. The lower priority requests are processed according to their priorities as the higher priority ISR bits are cleared.

In OCW3 ,the ESMM (enable special mask mode ) and SMM(special mask mode ) bits can be used to negate the priority modes in OCW2. If a byte is sent to OCW3 in  which both ESMM and SMM are set to 1 ,then unmasked interrupt requests are processed as they arrive and priority order is ignored.

Block diagram in Glenn A Gibson Text Book