1. What is Forward error correction (FEC) ?

Ans:

Forward error correction is a type of algorithm used in one-way data communications where additional data is added onto the data packet to allow the receiving end ensure that no errors are received. A system of error control for data transmission where in the receiving device has the capability to detect and correct any character or code block that contains fewer than a predetermined number of symbols in error. FEC is accomplished by adding bits to each transmitted character or code block, using a predetermined algorithm.

2. What is Convolution code ?

Ans:

Convolution code is a type of error correction code in which (a) each m-bit information symbol (each m-bit string) to be encoded is transformed into an n-bit symbol, where m/n is the code rate (n >= m) and (b) the transformation is a function of the last k information symbols, where k is the constraint length of the code.

6. Define Hamming Distance

Ans:

The number of bits which differ between two binary strings. More formally, the distance between two strings A and B is ∑ | Ai - Bi |. In comparing two bit patterns, the Hamming distance is the count of bits different in the two patterns. More generally, if two ordered lists of items are compared, the Hamming distance is the number of items that do not identically agree

The Hamming distance H is defined only for strings of the same length. For two strings s and t, H(s, t) is the number of places in which the two string differ, i.e., have different characters.

A measure of the difference between two messages, each consisting of a finite string of characters, expressed by the number of characters that need to be changed to obtain one from the other. E.g., 0101 and 0110 has a Hamming distance of two whereas "Butter" and "ladder" are four characters apart.

7. Write short noise on  ASCII code .

Ans:

ASCII - The American Standard Code for Information Interchange is a standard seven-bit code. This code  was proposed by ANSI in 1963, and  was finalized in 1968. ASCII, pronounced "ask-key".

ASCII was established to achieve compatibility between various types of data processing equipment. It is the common code for microcomputer equipment.

The standard ASCII character set consists of 128 decimal numbers ranging from zero through 127 assigned to letters, numbers, punctuation marks, and the most common special characters.

In addition to the 128 standard ASCII codes there are other 128 that are known as extended ASCII, and that are platform-dependent.The ASCII character set is a standard that incorporates systems like Windows, some UNIX platforms and many applications. It includes many more local symbols and marked letters so that it can be used with no need of being redefined in many more languages:

8. Write short notes on Baudot code.

Ans:

The Baudot code, was invented by Emile Baudot in 1870. It is  a character set predating EBCDIC and ASCII and used originally and primarily on tele printers . In Baudot, characters are expressed using five bits.

Baudot's original code is known as International Telegraph Alphabet No1, and is no longer used. The 'baudot' code has been used extensively in telegraph systems.. Using five bits allowed 32 different characters. To accommodate all the letters of the alphabet and numerals, two of the 32 combinations were used to select alternate character sets. Because this is not enough combinations to communicate all letters, numbers and needed symbols, two codes are used to "shift" which character set is to be used for printing subsequent characters. Baudot teleprinters were essentially modified typewriters, so the shift codes performed the same function as the SHIFT LOCK key on the keyboard, mechanically selecting one set of characters or the other.

The Baudot code is the first truly digital one. It consists of only two states, both logically and electrically. Logically, the only states are "1" and "0". Today we call the unit that can have these two logical states a "bit". Electrically, the states are current flowing (or "1"), and no current flowing (or "0"). Each character consists of 5 bits. Two logical states and five bits allows 32 characters. Baudot needed 26 characters for the alphabet, 10 for numbers, and more for miscellaneous characters. In order to increase the number of characters, he used two special characters, LTRS and FIGS, to give a total of 64 possibilities. LTRS (11111) precedes Alphabetic characters. FIGS (11011) precedes numerical and special characters such as punctuation. Baudot code was then improved by Donald Murray by adding extra characters and shift codes. This code is what is generally known as the 'Baudot code', also known as the International Telegraph Alphabet No 2 (ITA2). ITA2 is still used in some ham radio applications

9. Explain Go-Back N ARQ.

Ans:

In Go-Back-N ARQ, we can send up to W frames before worrying about acknowledgments. It  keeps a copy of these frames until the acknowledgments arrive. Frames from a sending station are numbered sequentially. A sequence number for each frame in the header is added. A limit is needed.  If the header of the frame allows m bits for the sequence number, the sequence numbers range from 0 to 2m - 1. At the sender site, to hold the outstanding frames until they are acknowledged, this ARQ uses a concept of a window. All the frames are stored in a buffer.  The frames to the left of the window are those that have already been acknowledged  those to the right of the window cannot be sent until the window slides over them. The size of the window is at most 2m - 1 The size of the window in this protocol is fixed. The window slides to include new unsent frames when the correct acknowledgments are received. The window is a sliding window.

The size of the window at the receiver site in this protocol is always 1. The receiver is always looking for a specific frame to arrive in a specific order.  Any frame arriving out of order is discarded and needs to be resent.

The sender has three variables, S, SF and S L The S variable holds the sequence number of the recently sent frame; SF holds the sequence number of the first frame in the window; and S L holds the sequence number of the last frame in the window. The size of the window is W,

where W = SL - SF + 1. The receiver only has one variable, R, that holds the sequence number of the frame it expects to receive.  If the sequence number of the received frame is the same as the value of R, the frame is accepted; if not, it is rejected. The sender sets a timer for each frame sent. The receiver has no timers. The receiver sends positive acknowledgments if a frame has arrived safe . If a frame is damaged or is received out of order, the receiver is  silent and will discard all subsequent frames until it receives the one it is expecting. If  silence of the receiver exceeds  timer limit. This, in turn, causes the sender to go back and resend all frames, beginning with the one with the expired timer.  The receiver does not have to acknowledge each frame received. It can send one cumulative acknowledgment for several frames.  Resending Frame When a frame is damaged, the sender goes back and sends a set of frames starting from the damaged one up to the last one sent. Suppose the sender has already sent frame 6, but the timer for frame 3 expires. This means that frame 3 has not been acknowledged,  so the sender goes back and sends frames 3, 4, 5, 6 again. That is why the protocol is called Go-Back-N ARQ.

Go-Back-N ARQ simplifies the process at the receiver site. The receiver keeps track of only one variable, and there is no need to buffer out-of-order frames; they are simply discarded. However, this protocol is very inefficient for a noisy link. In a noisy link a frame has a higher probability of damage, which means the resending of multiple frames. This resending uses up the bandwidth and slows down the transmission.

# 10. Explain Stop and Wait ARQ

Ans:

Stop and Wait transmission is the simplest reliability technique and is adequate for a very simple communications protocol. A stop and wait protocol transmits a Protocol Data Unit (PDU) of information and then waits for a response. The receiver receives each PDU and sends an Acknowledgement (ACK) PDU if a data PDU is received correctly. In practice, the receiver may not be able to reliably identify whether a PDU has been received, and the transmitter will usually also need to implement a timer to recover from the condition where the receiver does not respond.

Under normal transmission the sender will receive an ACK for the data and then commence transmission of the next data block. For a long delay link, the sender may have to wait an appreciable time for this response. While it is waiting the sender is said to be in the "idle" state and is unable to send further data.

Stop and Wait ARQ - Waiting for Acknowledgment (ACK) from the remote node.

When PDUs are lost, the receiver will not normally be able to identify the loss ,most receivers will not receive anything, not even an indication that something has been corrupted. The transmitter must then rely upon a timer to detect the lack of a response.

Stop and Wait ARQ - Retransmission due to timer expiry

In the diagram, the second PDU of Data is corrupted during transmission. The receiver discards the corrupted data. The sender is unaware of this loss, but starts a timer after sending each PDU. Normally an ACK PDU is received before this the timer expires. In this case no ACK is received, and the timer counts down to zero and triggers retransmission of the same PDU by the sender. The sender always starts a timer following transmission, but in the second transmission receives an ACK PDU before the timer expires, finally indicating that the data has now been received by the remote node.

- What is Manchester encoding ?

Ans:

Manchester encoding is a synchronous clock encoding technique used by the OSI physical layer to encode the clock and data of a synchronous bit stream. In this technique, the actual binary data to be transmitted over the cable are not sent as a sequence of logic 1's and 0's. Instead, the bits are translated into a slightly different format that has a number of advantages over using straight binary encoding.

In the Manchester Code, there is a transition at the middle of each bit period. The mid-bit transition serves as both a clocking mechanism and as data. A low to high transition represents a 1 and a high to low transition represents a 0.

Manchester Encoding has no DC component and there is always a transition available for synchronizing receive and transmit clocks. Because of the continuous presence of these transitions, Manchester Encoding is also called a self-clocking code.

| Original Data | Value Sent |
| --- | --- |
| Logic 0 | 0 to 1 (upward transition at bit centre) |
| Logic 1 | 1 to 0 (downward transition at bit centre) |

12.  How error detection and correction is achieved with Hamming code.

Ans:

Hamming code is an  error correction code that separates the bits holding the original value (data bits) from the error correction bits (check bits), and the difference between the calculated and actual error correction bits is the position of the bit that's wrong. If more than 1 bit changes, it is not possible to correct the error. Hamming code is a  1-bit error correction code, that can only correct 1-bit changes.

The transmitting station must append information to the data in the form of error correction bits, but the increase in frame length. Hamming codes provide for FEC using a "block parity" mechanism that can be inexpensively implemented.

The fundamental principal embraced by Hamming codes is parity. Hamming codes mentioned before, are capable of correcting one error or detecting two errors but not capable of doing both simultaneously. You may choose to use Hamming codes as an error detection mechanism to catch both single and double bit errors or to correct single bit error. This is accomplished by using more than one parity bit, each computed on different combination of bits in the data.

The number of parity or error check bits required is given by the Hamming rule, and is a function of the number of bits of information transmitted. The Hamming rule is expressed by the following inequality:

p

d + p + 1 < = 2

Where d is the number of data bits and p is the number of parity bits

13. Explain the principle of stop and wait ARQ ?

Ans:

Stop-and-wait is an error correction technique in which the sender transmits a block of data and then waits for an acknowledgment before transmitting the next block. Under normal transmission the sender will receive an ACK for the data and then commence transmission of the next data block. For a long delay link, the sender may have to wait an appreciable time for this response. While it is waiting the sender is said to be in the "idle" state and is unable to send further data

If data is corrupted during transmission. The receiver discards the corrupted data  The sender is unaware of this loss, but starts a timer after sending each block of data. Normally an ACK  is received before this the timer expires. In this case no ACK is received, and the timer counts down to zero and triggers retransmission of the same block of data by the sender. The sender always starts a timer following transmission, but in the second transmission receives an ACK before the timer expires, finally indicating that the data has now been received by the remote node.

14.	 Write notes on Cyclic redundant check.

Ans:

*	The CRC is a very powerful but easily implemented technique to obtain data reliability. The CRC technique is used to protect blocks of data called Frames. Using this technique, the transmitter appends an extra n- bit sequence to every frame.. This holds redundant information about the frame that helps the transmitter detect errors in the frame. The CRC is one of the most used techniques for error detection in data communications. The technique gained its popularity because it combines three advantages:

- Extreme error detection capabilities.

- Little overhead.

- Ease of implementation.