1. Explain Pulse-Width Modulation

Ans:

The pulse-width modulation is one kind of PTM. The system have a fixed amplitude and starting time of each pulse, but the width of each pulse is proportional to the amplitude of the signal at that instant. A negative pulse width is not possible. Pulse-width modulation has the disadvantage, when compared with pulse- position modulation (PPM) that its pulses are of varying width and therefore of varying power content. This means that the transmitter must be powerful enough to handle the maximum-width pulses, although the average power transmitted is perhaps only half of the peak power. PWM still works if synchronization between transmitter and receiver fails, whereas pulse-position modulation does not.

2. Explain the Generation and demodulation of PWM

Ans:

Pulse-width modulation may be generated by applying trigger pulses (at the sampling rate) to control the starting time of pulses from a monostable multivibrator, and feeding in the signal to be sampled to control the duration of these pulses. The emitter-coupled monostable multivibrator  makes an excellent voltage-to-time converter, since its gate width is dependent on the  voltage to which the capacitor C is charged. If this voltage is varied in accordance with a signal voltage, a series of rectangular pulses

will be obtained, with widths varying as required. The circuit does the twin jobs of sampling and converting the samples into PWM. It will be recalled that the stable state for this type of multi vibrator is with T 1 OFF and T 2 ON. The applied trigger pulse switches  T 1 ON, whereupon the voltage at C 1 falls as T 1 now begins to draw collector current, the voltage at B2 follows suit and T 2 is switched OFF by regenerative action. As soon as this happens, however, C begins to charge up to the collector supply potential through R. After a time determined by the supply voltage and the RC time constant of the charging network, B2 becomes sufficiently positive to switch T 2 ON. T 1 is simultaneously switched OFF by regenerative action and stays OFF until the arrival of the next trigger pulse. The voltage that the base of  T 2 must reach to allow T 2 to turn

on is slightly more positive than the voltage across the common emitter resistor Rk.  This voltage depends on the current flowing through the circuit, which at the time is the collector current of T 1. Collector current depends on the base bias, which is governed by the instantaneous  changes in the applied signal voltage. The applied modulation voltage controls the voltage  to which B2 must rise to switch T 2 ON. Since this voltage rise is linear the  modulation voltage is seen to control the period of time during which  T2 is OFF.

The demodulation of pulse-width modulation is quite a simple process. PWM is  fed to an integrating circuit from which the signal emerges whose amplitude at any time is proportional to the pulse width at that time.

3. What are the benefits and shortcomings of TDM?

Ans:

The relative simplicity and low cost of TDM hardware devices makes it possible to build cost-effective networks of widely varying node count, geographic extent, capacity, and speed.

However, TDM is best utilized if everyone always has something to send, otherwise, it wastes time.  Time Division Multiplexing operates with low network efficiency.  For example, if one of the voice channels on our connection is inactive, the bandwidth dedicated to the channel is unavailable to the other users; therefore, the alloted time slot remains idle.

Another deficiency is TDM's inability to transport a dynamically varying combination of voice, fax, and LAN traffic.  Because the bandwidth is statically allocated, the momentarily high bandwidth requirements of one channel on a network cannot be met.  There is no way for one TDM channel to grab all of the network bandwidth, even for a short time, to expedite the transfer of a large amount of data.

4. State and explain Sampling theorem.

Ans:

The theorem states that, when converting from an analog signal to digital (or otherwise sampling a signal at discrete intervals), the sampling frequency must be greater than twice the highest frequency of the input signal in order to be able to reconstruct the original perfectly from the sampled version.

fsample >=2 fmax

The sampling frequency is also called the NYQUIST FREQUENCY.

Sampling at this rate will not result in any loss of information, but if you sample at less than this then you will not be able to reconstruct the signal as it first appeared.The theorem was first formulated by Harry Nyquist in 1928 , but was only formally proved by Claude E. Shannon in 1949

5. What is meant by phase difference between two signal ?

Ans:

Phase describes the difference in the start of the cycle of one signal to the start of the cycle of another. One signal acts as a reference signal, the other signal is the phased signal. A phased signal is created by slightly delaying it in order to cause its peaks and troughs to be out of sync with the reference signal. The level of non-synchronization is measured in degrees. If a signal is 180º out of phase, it means that as the reference signal reaches zero voltage following a peak, the phased signal begins. Thus as the reference signal is peaking. The figure below illustrates this more clearly.

Phasing is the result of creating a signal out of sync with a reference signal.

- Explain the advantages and disadvantages of Amplitude Modulation.

Ans:

In amplitude modulation, the strength (amplitude) of the carrier from a transmitter is varied according to how a modulating signal varies. Amplitude modulation results in three separate frequencies being transmitted: the original carrier frequency, a lower sideband (LSB) below the carrier frequency, and an upper sideband (USB) above the carrier frequency. The sidebands are "mirror images" of each other and contain the same intelligence. Each sideband occupies as much frequency space as the highest audio frequency being transmitted. AM has the advantages of being easy to produce in a transmitter and AM receivers are simple in design. Its main disadvantage is its inefficiency. About two-thirds of an AM signal's power is concentrated in the carrier, which contains no intelligence. One-third of the power is in the sidebands, which contain the signal's intelligence. Since the sidebands contain the same intelligence, however, one is essentially "wasted." Of the total power output of an AM transmitter, only about one-sixth is actually productive, useful output!

Other disadvantages of AM include the relatively wide amount of frequency space an AM signal occupies and its susceptibility to static and other forms of electrical noise. Despite this, AM is simple to tune on ordinary receivers, and that is why it is used for almost all shortwave broadcasting.

- Draw the block diagram of a PCM system.

Ans: AND gate

Quantizer

Clock signal

Information Signal   Sampled

Signal ( PAM )  	  	Quantised data

Encoder

PCM signal

8.	Bring out the difference between analogue pulse modulation and digital pulse modulation with examples for each. (12)

Ans:

Pulse modulation may be used to transmit analog information, such as continuous speech or data. It is a system in which continuous waveforms are sampled at regular intervals. Information regarding the signal is transmitted only at the sampling times, together with any synchronizing pulses that may be required. At the receiving end, the original waveforms may be reconstituted from the information regarding the samples, if these are taken frequently enough. Despite the fact that information about the signal is not supplied continuously, as in AM and FM, the resulting receiver output can have negligible distortion.

Pulse modulation may be subdivided broadly into two categories, analog and digital. In analogue the indication of sample amplitude may be infinitely variable, while in the latter a code which indicates the sample amplitude to the nearest predetermined level is sent.

Pulse-amplitude and pulse-time modulation are both analog, while the pulse-code and delta modulation systems are both digital. All the modulation systems have sampling in common, but they differ from each other in the manner of indicating the sample amplitude.

The two types of analog pulse modulation, pulse-amplitude and pulse-time modulation correspond roughly to amplitude and frequency modulation.

Pulse-amplitude modulation (PAM) Pulse-amplitude modulation, the simplest form of pulse modulation. PAM is a pulse modulation system in which the signal is sampled at regular intervals, and each sample is made proportional to the amplitude of the signal at the instant of sampling.

Pulse-time modulation (PTM)  In PTM the signal is sampled as before, but the pulses indicating instantaneous sample amplitudes themselves all have a constant amplitude. One of their timing characteristics is varied, being made proportional to the sampled signal amplitude at that instant. The variable characteristic may be the width, position of frequency of the pulses, so that three different types of PTM are possible.

Pulse-Width Modulation

The pulse-width modulation of PTM is also often called PDM (pulse- duration modulation). In this it have a fixed amplitude and starting time of each pulse, but the width of each pulse is made proportional to the amplitude of the signal at that instant.

- Pulse-Position Modulation (PPM) The amplitude and width of the pulses is kept constant in this system, while the position of each pulse, in relation to the position of a recurrent reference pulse is varied by each instantaneous sampled value of the modulating. wave. This means that the transmitter must send synchronizing pulses to operate timing circuits in the receiver.

- Pulse-Code Modulation (PCM) In  Pulse-code modulation is the signal was sampled and sent in pulse form.  All the above  were forms of analog communication, in all these forms a signal is sent which has a characteristic that is infinitely variable and proportional to the modulating voltage. In common with the other forms of pulse modulation, PCM also uses the sampling technique, but it differs from the others in that it is a digital process. That is, instead of sending a pulse train capable of continuously varying one of the parameters, the PCM generator produces a series of numbers, or digits ,hence the name digital process. Each one of these digits, almost always in binary code, represents the approximate amplitude of the signal sample at that instant. The approximation can be made as close as desired, but it is always just that, an approximation.