1. What is Circuit Switching ?

Ans:

In circuit-switched networks, network resources are static, set in “copper” if you will, from the sender to receiver before the start of the transfer, thus creating a “circuit”. The resources remain dedicated to the circuit during the entire transfer and the entire message follows the same path. A ``dedicated'' circuit is set up for each connection. The communicating parties use this fixed circuit during the conversation. Circuit switching is a process that establishes connections on demand and permits exclusive use of those connections until they are released. Once the communication is finished, the circuit can be released for other uses.

Circuit switching was developed to handle voice traffic but it can also handle digital data transmition but for that it is not so efficieant.

Public telephone network uses this technology.

2. State the Advantages and disadvantages of circuit switching ?

Ans:

Advantages:

- Fixed bandwidth, guaranteed capacity (no congestion)

- Low varience end-to-end delay (delay is almost constant)

Disadvantages:

- Connection set-up and tear-down introduces extra overhead (thus initial delay)

- User pay for circuit, even when not sending data

- Other users can't use the circuit even if it is free of traffic (statistics show that during a typical phone conversation, 64-73% of the time one speaker talking, 3-7% of the time both spearkers talking, 20-33% of the time both speakers silent).

## 3. Explain Isochronous Transmission

Ans:

- The ISOCHRONOUS (ISOC) format for data transmission is a procedure or protocol in

which each information CHARACTER or BYTE is individually synchronized or FRAMED by the use of Start and Stop Elements, also referred to as START BITS and STOP BITS.

The Isochronous Transmission Format is also known as START-STOP mode or CHARACTER mode. Each character or byte is framed as a separate and independent unit of DATA that may be transmitted and received at irregular and independent time intervals. The characters or bytes may also be transmitted as a contiguous stream or series of characters.

The character or byte may contain the number of bits required to allow translation of the BIT PATTERN into a group of symbols used to represent:

- LETTERS (alpha characters)

- NUMBERS (numerical values)

- PUNCTUATION MARKS

- CONTROL ELEMENTS

## Elements of an Isochronous Data Communication Network

TERMINAL     MODEM                    MODEM      TERMINAL

_____       _____   COMMUNICATIONS   _____       _____

|     |     |     |       LINK       |     |     |     |

| DTE |-<=>-| DCE |__/\  /\  /\  /\__| DCE |-<=>-| DTE |

|     |  ^  |     |    \/  \/  \/    |     |  ^  |     |

|_____|  |  |_____|       DATA       |_____|  |  |_____|

^     |         `--- CLOCKING ---'         |     ^

|     |                                    |     |

|     `------------- INTERFACE ------------'     |

|                                                |

`---------------- EXTERNAL  CLOCK ---------------'

If clocking is NOT provided by the modems (DCE), refer to the Asynchronous Transmission Overview.

The TERMINALS or DTE devices normally communicate with other terminals or DTE devices across a communications NETWORK via some form of MODEMS (MOdulator-DEModulators) that are connected through a communications LINK.

The terminals are connected to the modems through an INTERFACE. There are many different types of interfaces in use due to the differences in the characteristics of the DTE terminals and the communications links being used, and the performance requirements.

The Isochronous interface normally will include some form of timing, data STROBE, or CLOCK that will be used to ensure a steady and continuous flow of data.

The terminals or DTE devices operating in the Isochronous Mode will normally require EXTERNAL TIMING or CLOCKS to strobe the data out of and into the modems or DCE Devices.

The modems or DCE devices operating ASYNC normally provide any data timing or clocks to the DTE devices.

## Isochronous Character Format

THE LETTER "A" =  (HEX 41) USING ASCII 7 DATA BIT EVEN PARITY

MSB                         LSB       SPACE = + V

___     ___     ___________________     ___

|   |   |   |   |   :   :   :   :   |   |   |

| S | S | 0 | 1 | 0 . 0 . 0 . 0 . 0 | 1 | S |  <--- IDLE

____|   |___|   |___|   .   .   .   .   |___|   |____________

^   ^                                   ^

1   |   |   8   4   2   1   8   4   2   1   |    MARK= - V

|   |                                   |

START BIT   |         HEX = 4  +  1             `--- START BIT

STOP BIT         COL + ROW                  (1 BIT)

The order of transmission is from LSB to MSB.

THE LETTER "R" =  (HEX D2) USING ASCII 7 DATA BIT EVEN PARITY

MSB                         LSB       SPACE = + V

___    .   .    ___     _______     ___ ___

|   |   .   .   |   |   |   :   |   |   :   |

| S | S . 1 . 1 | 0 | 1 | 0 . 0 | 1 | 0 . S |  <--- IDLE

____|   |___:___:___|   |___|   .   |___|   .   |_____________

^   ^                                   ^

|   |   8   4   2   1   8   4   2   1   |    MARK= - V

|   |                                   |

START BIT   |         HEX = D  +  2             `--- START BIT

STOP BIT         COL + ROW                  (1 BIT)

4. Explain  asynchronous transmission ?

Ans:

The ASYNCHRONOUS (ASYNC) format for data transmission is a procedure or protocol in which each information CHARACTER or BYTE is individually synchronized or FRAMED by the use of Start and Stop Elements, also referred to as START BITS and STOP BITS.

The Asynchronous Transmission Format is also known as START-STOP mode or CHARACTER mode. Each character or byte is framed as a separate and independent unit of DATA that may be transmitted and received at irregular and independent time intervals. The characters or bytes may also be transmitted as a contiguous stream or series of characters.

The character or byte may contain the number of bits required to allow translation of the BIT PATTERN into a group of symbols used to represent:

- LETTERS (alpha characters)

- NUMBERS (numerical values)

- PUNCTUATION MARKS

- CONTROL ELEMENTS

## Elements of an Asynchronous Data Communication Network

TERMINAL     MODEM                    MODEM      TERMINAL

_____       _____   COMMUNICATIONS   _____       _____

|     |     |     |       LINK       |     |     |     |

| DTE |-<=>-| DCE |__/\  /\  /\  /\__| DCE |-<=>-| DTE |

|     |  ^  |     |    \/  \/  \/    |     |  ^  |     |

|_____|  |  |_____|                  |_____|  |  |_____|

^     |         `--- NO CLOCK ---'         |     ^

|     |                                    |     |

|     `------------- INTERFACE ------------'     |

|                                                |

`---------------- INTERNAL  CLOCK ---------------'

If clocking is provided by the modems (DCE), refer to the Isochronous Transmission Overview.

The TERMINALS or DTE devices normally communicate with other terminals or DTE devices across a communications NETWORK via some form of MODEMS (MOdulator-DEModulators) that are connected through a communications LINK.

The terminals are connected to the modems through an INTERFACE. There are many different types of interfaces in use due to the differences in the characteristics of the DTE terminals and the communications links being used, and the performance requirements.

The terminals or DTE devices operating in the Asynchronous Mode will normally provide the INTERNAL TIMING or CLOCKS required to strobe the data out of and into the modems or DCE Devices.

The modems or DCE devices operating ASYNC normally do not provide any data timing or clocks.

## Asynchronous Character Format

THE LETTER "A" =  (HEX 41) USING ASCII 7 DATA BIT EVEN PARITY

MSB                         LSB       SPACE = + V

___     ___________________     ___

|   |   |   :   :   :   :   |   |   |

<--  S  S | 0 | 1 | 0 . 0 . 0 . 0 . 0 | 1 | S |  <--- IDLE

___________|   |___|   .   .   .   .   |___|   |_____________

^  ^                                   ^

|  |   8   4   2   1   8   4   2   1   |    MARK= - V

\__/                                   |

STOP BITS '          HEX = 4  +  1              `--- START BIT

(1 or 2 BITS)             COL + ROW                   (1 BIT)

The order of transmission is from LSB to MSB.

THE LETTER "R" =  (HEX D2) USING ASCII 7 DATA BIT EVEN PARITY

MSB                         LSB       SPACE = + V

.   .    ___     _______     ___ ___

.   .   |   |   |   :   |   |   :   |

<--  S  S . 1 . 1 | 0 | 1 | 0 . 0 | 1 | 0 . S |  <--- IDLE

___________:___:___|   |___|   .   |___|   .   |_____________

^  ^                                   ^

|  |   8   4   2   1   8   4   2   1   |    MARK= - V

\__/                                   |

STOP BITS '          HEX = D  +  2              `--- START BIT

(1 or 2 BITS)             COL + ROW                   (1 BIT)

5. What is Shot noise ?

Ans:

Shot noise refers to the random fluctuations of the electric current in an electrical conductor, which are caused by the fact that the current is carried by discrete charges (electrons). Short noice arices in electronic divices such as diode and transistors.The strength of this noise increases for growing magnitude of the average current flowing through the conductor.

Shot noise is important in electronics, telecommunication, and for fundamental physics. The strength of the current fluctuations can be expressed by giving the variance of the current. However, the value measured in this way depends on the frequency range of fluctuations which is measured ("bandwidth" of the measurement): The measured variance of the current grows linearly with bandwidth. Therefore, a more fundamental quantity is the noise power, which is essentially obtained by dividing through the bandwidth (and, therefore, has the dimension ampere squared divided by Hertz).

6. What is Signal to Noise Ratio ?

Ans:

Signal to Noise Ratio is defined as the ratio of average signal power to the average noise power, both being measured at the same point. SNR is expressed in decibels. (dB)

In analog and digital communications, signal-to-noise ratio is often written S/N or SNR.

If the incoming signal strength in microvolts is Vs, and the noise level, also in microvolts, is Vn, then the signal-to-noise ratio, S/N, in decibels is given by the formula

S/N = 20 log10(Vs/Vn)

7. Explain Thermal Noise

Ans:

Thermal noise is the electrical noise arising from the random motion of electrons in a conductor.

Thermal noise also called Johnson noise is generated by thermal agitation of electrons in a conductor. The noise power, P , in watts, is given by P = kT Δf , where k is Boltzmann's constant in joules per kelvin, T is the conductor temperature in kelvins, and Δf is the bandwidth in hertz.

- Thermal noise power, per hertz, is equal throughout the frequency spectrum, depending only on k and T .

- For the general case, the above definition may be held to apply to charge carriers in any type of conducting medium.

The mean – square value of the thermal noise voltage  VTN appearing across the terminals of a resistor, measured in a bandwidth of      f  Hertz is given by

E[V2TN] = 4kTR    f volts 2

Where k is the Boltzmann’s constant equal to 1.38* 10-23 joules per degree Kelvin

T is the absolute temperature in degree Kelvin, and R is the resistance in ohms.

8. Explain Message Switching

Ans:

Message Switching works similar to the postal department.. In this technology, an entire message (analogous to a letter) is routed to a destination (recipient). The message will follow several paths and decisions as to the proper pathway must also be made. Once the letter is processed locally, a decision has to be made as the most expeditious manner of getting to a post office near its destination. Like that the entire message block  held until it is processed and the proper route for delivering it is selected. This process continues till it reaches the final destination as mentioned.

So we can call it as a  store and forward method. In this scheme, messages are received in their entirety and stored while a route to or closer to their destination is being determined. Once the route has been established, then the message is sent to either its final destination or another intermediate storage and forwarding point.

One of the prime users of this type of technology is electronic mail systems. In E-mail networks, all devices storing E-mail (typically called E-mail or mail servers) may not be connected all of the time. They may only connect with one another intermittently. No exclusive pathway is required to get the message from Point A to Point B, only a series of links that are created as needed. The message itself contains information as to where it is going. The nodes along the way temporarily store the message and select the next best route to send it on, maintain a listing of those pathways. The message is stored as many times as is necessary to forward it to the right place.

The machines (or nodes) that are used in message switching are not really special machines other than the fact that they generally require a lot of disk space to accommodate long messages. Large hard disks can get expensive, so there is an economic factor to this technology.

A bad point from a wide area network point of view is that storing and forward on a message switched system is slow. Obviously to store a whole message, select a route, and then retransmit the message takes time. This mechanism is totally unsuitable for communicating in real time.

In summary, message switching offers good use of network resources providing several features stemming from the storage of messages along the way. However, the expense and delay of the store and forward schema are not practical some time.

9. What are the modes of Channel Operation?

Ans:

Simplex
Data in a simplex channel is always one way. Simplex channels are not often used because it is not possible to send back error or control signals to the transmit end.

It's like a one way street. An example of simplex is Television, or Radio

Half Duplex

A half-duplex channel can send and receive, but not at the same time. It's like a one-lane bridge where two way traffic must give way in order to cross. Only one end transmits at a time, the other end receives. In addition, it is possible to perform error detection and request the sender to retransmit information that arrived corrupted.

Another example of half-duplex is walky – talky

Full Duplex

Data can travel in both directions simultaneously. There is no need to switch from transmit to receive mode like in half duplex. Its like a two lane bridge on a two-lane highway. 
An example is the telephone system.

10.  What is partition noise ?

Ans:

Partition noise:

If two particles collide and scatter, the process that results in the detection of the first particle in one direction and the second particle in another direction. This interference is the origin of the Pauli exclusion principle, which states that two electrons can never occupy the same state. The scattering of individual particles is  a  process that introduces fluctuations called partition noise.

11. 	What is a packet ? Explain packet switching.

Ans:

Any message exceeding a network-defined maximum length is broken up into shorter units, known as packets, for transmission; the packets, each with an associated header, are then transmitted individually through the network. The fundamental difference in packet communication is that the data is formed into packets with a pre-defined header format

In packet switched data networks all data to be transmitted is first assembled into packets by the source DTE. These packets include both the source and the destination DTE network addresses. They are then passed by the source DTE to the network ie to its local Packet Switching Exchange (PSE). On receipt of each packet, the PSE inspects the destination address contained in the packet. Each PSE contains a routing directory specifying the outgoing links to be used for each network address. On receipt of each packet, the PSE forwards the packet on the appropriate link at the maximum available bit rate. As each packet is received at each intermediate PSE along the route, it is forwarded on the appropriate link interspersed with other packets being forwarded on that link. At the destination PSE, determined by the destination address within the packet, the packet is finally passed to the destination DTE.

12.  What is circuit switching ?Explain its  advantage and disadvantages

Ans:

In circuit-switched networks, network resources are static, set in copper if you will, from the sender to receiver before the start of the transfer, thus creating a “circuit”. The resources remain dedicated to the circuit during the entire transfer and the entire message follows the same path. A dedicated circuit is set up for each connection. The communicating parties use this fixed circuit during the conversation. Circuit switching is a process that establishes connections on demand and permits exclusive use of those connections until they are released. Once the communication is finished, the circuit can be released for other uses.

Circuit switching was developed to handle voice traffic but it can also handle digital data transmition but for that it is not so efficieant.

Public telephone network uses this technology.

Advantages:

Fixed bandwidth, guaranteed capacity (no congestion)

Low varience end-to-end delay (delay is almost constant)

Disadvantages:

Connection set-up and tear-down introduces extra overhead (thus initial delay)

User pay for circuit, even when not sending data

Other users can't use the circuit even if it is free of traffic (statistics show that during a typical phone conversation, 64-73% of the time one speaker talking, 3-7% of the time both spearkers talking, 20-33% of the time both speakers silent).

13.  Bring out the difference between Synchronous, Asynchronous and Isochronous

transmission.

Ans:

Synchronous transmission is data transmission in which source and sink have a common view of time and both run clocks that are continually synchronized by coordinating pulses extracted from the incoming data stream. Synchronous Transmission sends packets of characters simultaneously. Each packet is preceded by a Start Frame. This frame is used to tell the receiving station that a new packet of characters is arriving, and also to synchronize the receiving station's internal clock. The packets also have End Frames to indicate the end of the packet. The packet can contain up to 64,000 bits, depending on the protocol. Both Start and End Frames have a special bit sequence (that the receiving station recognizes) to indicate the start and end of a packet: the Start and End Frames may be only 2 bytes each.

The ASYNCHRONOUS (ASYNC) format for data transmission is a procedure or protocol in which each information CHARACTER or BYTE is individually synchronized or FRAMED by the use of Start and Stop Elements, also referred to as START BITS and STOP BITS.

The Asynchronous Transmission Format is also known as START-STOP mode or CHARACTER mode. Each character or byte is framed as a separate and independent unit of DATA that may be transmitted and received at irregular and independent time intervals. The characters or bytes may also be transmitted as a contiguous stream or series of characters.

Synchronous transmission is more efficient than asynchronous (character transmission). For example, only 4 bytes (2 Start Framing Bytes and 2 Stop Framing bytes) are required to transmit up to 8K bytes. Extra bytes (e.g. the Start and Stop Frame) that are not part of the data are called overhead. Packet overhead consists of control information that is used to control the communication.

The principle difference between the synchronous and asynchronous modes of transmission is that in the synchronous case, the receiver uses a clock which is synchronised to the transmitter clock.

TERMINAL     MODEM                    MODEM      TERMINAL

_____       _____   COMMUNICATIONS   _____       _____

|     |     |     |       LINK       |     |     |     |

| DTE |-<=>-| DCE |__/\  /\  /\  /\__| DCE |-<=>-| DTE |

|     |  ^  |     |    \/  \/  \/    |     |  ^  |     |

|_____|  |  |_____|                  |_____|  |  |_____|

^     |         `--- NO CLOCK ---'         |     ^

|     |                                    |     |

|     `------------- INTERFACE ------------'     |

|                                                |

`---------------- INTERNAL  CLOCK ---------------'

The ISOCHRONOUS (ISOC) format for data transmission is a procedure or protocol in which each information CHARACTER or BYTE is individually synchronized or FRAMED by the use of Start and Stop Elements, also referred to as START BITS and STOP BITS.

The Isochronous Transmission Format is also known as START-STOP mode or CHARACTER mode. Each character or byte is framed as a separate and independent unit of DATA that may be transmitted and received at irregular and independent time intervals. The characters or bytes may also be transmitted as a contiguous stream or series of characters.

TERMINAL     MODEM                    MODEM      TERMINAL

_____       _____   COMMUNICATIONS   _____       _____

|     |     |     |       LINK       |     |     |     |

| DTE |-<=>-| DCE |__/\  /\  /\  /\__| DCE |-<=>-| DTE |

|     |  ^  |     |    \/  \/  \/    |     |  ^  |     |

|_____|  |  |_____|       DATA       |_____|  |  |_____|

^     |         `--- CLOCKING ---'         |     ^

|     |                                    |     |

|     `------------- INTERFACE ------------'     |

|                                                |

`---------------- EXTERNAL  CLOCK ---------------'

If clocking is NOT provided by the modems (DCE), refer to the Asynchronous Transmission Overview. Isochronous usually it means "self-clocked".  That is, there is a given speed at which the channel operates, and there is no separate clock signal.  Thus it is like a synchronous line in that there is no start-stop function, but unlike a sync DTE interface, there is no clock lead.