1.  Classify Modems according to  Synchronization

Ans:

Asynchronous Modems

Most of the modems that operate in slow and moderate rates, up to 1800 bps, are asynchronous (using asynchronous data). Asynchronous modems operate in FSK modulation and use two frequencies for transmission and another two for receiving.

Asynchronous data

Asynchronous data is not accompanied by any clock, and the transmitting and receiving modems know only the nominal data rate. To prevent slipping of the data relative to the modems' clocks, this data is always grouped in very short blocks (characters) with framing bits (start and stop bits). The most common code used for this is the seven-bit ASCII code with even parity.

Synchronous Modems

Synchronous modems operates in the audio domain, at rates up to 28800 bps in audio lines, used in telephones systems (using synchronous data). The usual modulation methods are the phase modulation and integrated phase and amplitude (at higher rates than 4800 bps).

In synchronous modems, equalizers are used, in order to offset the misfit of the telephone lines. These equalizers are inserted in addition to the equalizers, that sometimes already exist in the telephone lines.

Synchronous modems operate in the same manner asynchronous modems. However, synchronous modems operates at higher rates and since the requirements to transmit at these rates is increasing, most of the innovations are implemented for synchronous modems.

In synchronous modems the channel can be split for several consumers at various speeds. Modems who have this ability are called SSM - Split System Modem. These modems can use a simple split or a split using multipoint connection.

Synchronous data is accompanied by a clock signal. Synchronous data is almost always grouped in blocks, and it is the responsibility of the data source to assemble those blocks with framing codes and any extra bits needed for error detecting and/or correcting according to one of many different protocols (BISYNC, SDLC, HDLC, etc.). The data source and destination expect the modem to be transparent to this type of data, conversely, the modem can ignore the blocking of the data.

2. What is Short Haul modem ?

Ans:

Short haul modems are cheap solutions to systems of short ranges (up to 15 km), which use private lines and are not part of a public system. Short haul modems can also be used, even if the end-to-end length of the direct connection is longer than 15 km, when both ends of the line are served by the same central office in the telephone system. These lines are called "local loops". Short haul modems are distance-sensitive, because signal attenuation occurs as the signal travels through the line. The transmission rate must be lowered to ensure consistent and error-free transmission on longer distances.

There are two main types of short haul modems:

- Analog modems, using a simple modulation method, without sophisticated devices for error control or equalizers. These modems usually operates at a maximum rate of 9600 bps, but there are some which supports higher rates (up to 64,000 bps).

- Line drivers increase the digital signal, which transmit to the communication channel and do not transmit the carrier signal, as conventional modems. Line drivers are very cheap and tiny and connect to the RS232 connector of the terminal (since they lack a power supply, they use the signal voltage of the DTE-DCE interface for DC power supply).

3. What is  Voice Grade modem ?

Ans:

Voice-grade modems are used for unlimited destination, using a moderate to high data rate. These modems are expensive and their maintenance and tuning are sophisticated. Communication channels are leased lines and dial-up.

Voice-band telephone network is used for data transmission. A user-to-user connection may be either dedicated or dialed. The links in the connection are the same in the two cases, and the only difference for the user is that for some impairments (particularly attenuation and delay distortion), a dedicated (private or leased) line is guaranteed to meet certain specifications, whereas a dialed connection can only be described statistically.

- 4. What is Data Concentrator

Ans:

The  Data Concentrator acts as a Data Hub providing convenient connection points for external interfaces to the system. This is particularly important in the case where the RTUs are distributed around the substation and a central connection point is necessary to facilitate interfacing. The Data Concentrator provides the following functionality:

Interfaces
Modularity & Expansion
Redundancy
Time Synchronisation
Remote Communications

5. What is Protocol?

Ans:

When data is being transmitted between two or more devices something needs to govern the controls that keep this data intact. A formal description of message formats and the rules two computers must follow to exchange those messages. Protocols allow data to be taken apart for faster transmission, transmitted, and then reassembled at the destination in the correct order. The protocol used determines the way errors are checked, the type of compression, the way the sender indicates the end of the transmission, and the way the receiver indicates that the message has been received. Protocols can describe low-level details of machine-to-machine interfaces (e.g., the order in which bits and bytes are sent across a wire) or high-level exchanges between allocation programs (e.g., the way in which two programs transfer a file across the Internet). 
A set of formal rules describing how to transmit data, especially across a network. Low-level protocols define the electrical and physical standards to be observed, bit- and byte-ordering and the transmission and error detection and correction of the bit stream. High-level protocols deal with the data formatting, including the syntax of messages, the terminal-to-computer dialog, character sets, and sequencing of messages.

6. What is Front end processor ?

Ans:

The front end processor or FEP is a computer that handles input and output communications for large mainframe computer, reducing the computers load.

Front end processor also known as a communications controller

7. Distinguish between internet and intranet.

Ans:

An intranet is a private network that is contained within an enterprise. It may consist of many interlinked local area networks and also use leased lines in the Wide Area Network. Typically, an intranet includes connections through one or more gateway computers to the outside Internet. The main purpose of an intranet is to share company information and computing resources among employees. An intranet can also be used to facilitate working in groups and for teleconferences. 
An intranet uses TCP/IP, HTTP, and other Internet protocols and in general looks like a private version of the Internet

The Internet is a worldwide collection of computer networks connecting academic, governmental, commercial, and organizational sites. It provides access to communication services and information resources to millions of users around the globe. The underlying protocol with which these systems communicate is called TCP/IP. Any computer system directly connected to the network has a domain name and an IP (numeric) address. The domain names are typically of the form system.site.domain

Using the Web, you have access to millions of pages of information. Web browsing is done with a Web browser, the most popular of which are Microsoft Internet Explorer and Netscape Navigator.

The Internet is an open, public space, while an intranet is designed to be a private space. An intranet may be accessible from the Internet, but as a rule it's protected by a password and accessible only to employees or other authorized users.

- The tasks differ. The intranet is used for everyday work inside the company, including some quite complex applications; the Internet site is mainly used to find out information about your products.

- The type of information differs. The intranet will have many draft reports, project progress reports, human resource information, and other detailed information, whereas the Internet site will have marketing information and customer support information.

The amount of information differs. Typically, an intranet has between ten and a hundred times as many pages as the same company's public website. The difference is due to the extensive amount of work-in-progress that is documented on the intranet and the fact that many projects and departments never publish anything publicly.

8. Write notes on fiber optic communication.

Fiber optic communication

Fiber optic cable has the ability  to transmit more information, more quickly and over longer distances. Fiber optic cable offers almost unlimited bandwidth and unique advantages over all previously developed transmission media. The basic point-to-point fiber optic transmission system consists of three basic elements: the optical transmitter, the fiber optic cable and the optical receiver.

The Optical Transmitter: The transmitter converts an electrical analog or digital signal into a corresponding optical signal. The source of the optical signal can be either a light emitting diode, or a solid state laser diode. The most popular wavelengths of operation for optical transmitters are 850, 1300, or 1550 nanometers.

The Fiber Optic Cable: The cable consists of one or more glass fibers, which act as waveguides for the optical signal. Fiber optic cable is similar to electrical cable in its construction, but provides special protection for the optical fiber within. For systems requiring transmission over distances of many kilometers, or where two or more fiber optic cables must be joined together, an optical splice is commonly used.

The Optical Receiver: The receiver converts the optical signal back into a replica of the original electrical signal. The detector of the optical signal is either a PIN-type photodiode or avalanche-type photodiode.

9. Write short notes on  Bridges.

Ans:

A bridge is a simple device which  split the physical network being shared by many computers into smaller segments. A bridge generally has only two ports; bridges with more than two ports are usually called switches. Bridges are normally used to connect LAN segments within a limited geographic area (local bridges), like a building or a campus. Bridges can be programmed to reject packets from particular networks.

Ethernet is the most commonly used physical network. On an Ethernet network, all the connected computers share the same piece of "wire" (it's not physically the same piece, but it is electrically). When two computers attempt to talk at the same time, they drown each other out and create what's called a collision. The more computers you have on one Ethernet, the bigger chance you'll have a collision.

Bridges split an Ethernet into multiple collision-domains. All the data on one side of the bridge stays there, unless it's destined for a computer on the other side of the bridge, lessening the overall load on each segment. Bridges forward all broadcast messages. Only a special bridge called a translation bridge will allow two networks of different architectures to be connected. Bridges do not normally allow connection of networks with different architectures. It does not propagate noise signals and defective frames as it was the case for repeaters (at the physical layer). It adaptively recognizes which machines are reachable from a port. It reduces traffic on each port and it improves security since each port will only transmit frames directed to nodes reachable from that port. All the nodes reacheable from a node through segments and bridges will receive broadcast messages sent by that node.

Bridges don't care what protocol is being used on the network (TCP/IP, IPX, AppleTalk, etc.) since they operate at  the data-link level. This is both a benefit and a curse; since they work at such a simple level, bridges are able to operate at blindingly fast speeds, but since they will indiscriminately forward data, one has little control over their operation. This is where routers come in.

10. Write Short notes on Routers

Ans:

A router is a box or a  regular computer with at least two ports, used to connect also dissimilar networks. A router is used to route data packets between two networks. It reads the information in each packet to tell where it is going. It differs from bridges since it operates at the network level. It will also use different addresses. For example a bridge may use Ethernet addresses while a router uses IP addresses. Routers work at the network layer - they actually understand the protocols being used to carry the data over the network. And since they understand the protocols, they can use rules to decide what to do with a specific piece of data. Because of this, routers are useful in linking networks that are used for different purposes or by different organizations. One can apply rules or filters to let certain data in, but keep other data out. Or to route data serving one purpose over a certain set of network connections, while routing other data over other connections. This convenience comes at a price. The more detail a router must acquire about a specific piece of data before forwarding it on, the longer that piece of data is delayed before being sent on to its destination. Also, the greater configurability of routers requires faster, more expensive hardware.

It does all the transformations that may be required by the transfer of packets across the networks it connects. Routing involves two basic activities: running routing algorithms to determine routes, as expressed by routing tables, and using the routing tables to move packets across the network. The latter activity is easier and it is called switching. Routing tables contain information that will indicate for each packet on the basis of its final destination (usually an IP address) where to go next (next-hop forwarding) as to the port to be used and the physical address of the next router.

- 11. Write short notes on Gateways

Ans:

- A gateway can translate information between different network data formats or network architectures. It can translate TCP/IP to AppleTalk so computers supporting TCP/IP can communicate with Apple brand computers. Most gateways operate at the application layer, but can operate at the network or session layer of the OSI model. Gateways will start at the lower level and strip information until it gets to the required level and repackage the information and work its way back toward the hardware layer of the OSI model.

### 12. Write short note on  Network Repeater ?

Ans:

A repeater connects two segments of your network cable. It retimes and regenerates the signals to proper amplitudes and sends them to the other segments. When talking about, ethernet topology, you are probably talking about using a hub as a repeater. Repeaters require a small amount of time to regenerate the signal. This can cause a propagation delay which can affect network communication when there are several repeaters in a row. Many network architectures limit the number of repeaters that can be used in a row. Repeaters work only at the physical layer of the OSI network model.

13. Write short notes Internetwork.

Ans:

An internetwork is a collection of individual networks, connected by intermediate networking devices, that functions as a single large network. Internetworking refers to the industry, products, and procedures that meet the challenge of creating and administering internetworks.Internetworking evolved as a solution to three key problems: isolated LANs, duplication of resources, and a lack of network management. Isolated LANs made electronic communication between different offices or departments impossible. Duplication of resources meant that the same hardware and software had to be supplied to each office or department, as did separate support staff. This lack of network management meant that no centralized method of managing and troubleshooting networks existed.

To implementing a functional internetwork many challenges must be faced, especially in the areas of connectivity, reliability, network management, and flexibility.

14. Write Short note on SONET ?

Ans:

SONET (Synchronous Optical Network) - SONET is the U.S. (ANSI) standard for synchronous data transmission on optical media.  SONET provides standards for a number of line rates up to the maximum line rate of 9.953 gigabits per second (Gbps).  SONET is considered to be the foundation for the physical layer of the broad band ISDN. It uses the Time Division Multiplexing.

15. Mention some advantages of fiber optic communication over the traditional methods.

Ans:

Fiber optic transmission systems – a fiber optic transmitter and receiver, connected by fiber optic cable – offer a wide range of benefits not offered by traditional copper wire or coaxial cable. These include:

1. The ability to carry much more information and deliver it with greater fidelity than either copper wire or coaxial cable.

2. Fiber optic cable can support much higher data rates, and at greater distances, than coaxial cable, making it ideal for transmission of  serial digital data.

3. The fiber is totally immune to virtually all kinds of interference, including lightning, and will not conduct electricity. It can therefore   come in direct contact with high voltage electrical equipment and power lines. It will also not create ground loops of any kind.

4. As the basic fiber is made of glass, it will not corrode and is unaffected by most chemicals. It can be buried directly in most kinds of  soil or exposed to most corrosive atmospheres in chemical plants without significant concern.

5. Since the only carrier in the fiber is light, there is no possibility of a spark from a broken fiber. Even in the most explosive of atmospheres, there is no fire hazard, and no danger of electrical shock to personnel repairing broken fibers.

6. Fiber optic cables are virtually unaffected by outdoor atmospheric conditions, allowing them to be lashed directly to telephone poles or existing electrical cables without concern for extraneous signal pickup.

7. A fiber optic cable, even one that contains many fibers, is usually much smaller and lighter in weight than a wire or coaxial cable with similar information carrying capacity. It is easier to handle and install, and uses less duct space. (It can frequently be installed without ducts.)

8. Fiber optic cable is ideal for secure communications systems because it is very difficult to tap but very easy to monitor. In addition, there is absolutely no electrical radiation from a fiber

16. Write short note on fiber optic communication.

Ans:

Fiber optic cable has the ability  to transmit more information, more quickly and over longer distances. Fiber optic cable offers almost unlimited bandwidth and unique advantages over all previously developed transmission media. The basic point-to-point fiber optic transmission system consists of three basic elements: the optical transmitter, the fiber optic cable and the optical receiver.

The Optical Transmitter: The transmitter converts an electrical analog or digital signal into a corresponding optical signal. The source of the optical signal can be either a light emitting diode, or a solid state laser diode. The most popular wavelengths of operation for optical transmitters are 850, 1300, or 1550 nanometers.

The Fiber Optic Cable: The cable consists of one or more glass fibers, which act as waveguides for the optical signal. Fiber optic cable is similar to electrical cable in its construction, but provides special protection for the optical fiber within. For systems requiring transmission over distances of many kilometers, or where two or more fiber optic cables must be joined together, an optical splice is commonly used.

The Optical Receiver: The receiver converts the optical signal back into a replica of the original electrical signal. The detector of the optical signal is either a PIN-type photodiode or avalanche-type photodiode.

17. Write notes on coaxial cable.

Ans:

Coaxial cable consists of two conductors. The inner conductor is held inside an insulator with the other conductor woven around it providing a shield. An insulating protective coating called a jacket covers the outer conductor.

The outer shield protects the inner conductor from outside electrical signals. The distance between the outer conductor (shield) and inner conductor plus the type of material used for insulating the inner conductor determine the cable properties or impedance. Typical impedances for coaxial cables are 75 ohms for Cable TV, 50 ohms for Ethernet. The excellent control of the impedance characteristics of the cable allow higher data rates to be transferred than with twisted pair cable.

18. Differentiate between   Internal & External modems.

Ans:

External modems sit next to the computer and connect to the serial port using a straight-through serial cable.

An internal modems is a plug-in circuit board that sits inside the computer. It incorporates the serial port on-board. They are less expensive than external modems because they do not require a case, power supply and serial cable. They appear to the communication programs as if they were an external modem for all practical purposes.

## 19. Discuss on different types of modem.

Ans:

There are many types of modems, the most common of which are:

- Optical Modem - Uses optical fiber cable instead of wire. The modem converts the digital signal to pulses of light to be transmitted over optical lines (more commonly called a media adapter or transceiver).

- Short Haul Modem - A modem used to transmit data over 20 miles or less. Modems we use at home or to connect computers together among different offices in the same building are short haul modems.

- Acoustic Modem - a modem that couples to the telephone handset with what looks like suction cups that contain a speaker and microphone. Used by traveling salespeople to connect to hotel phones.

- Smart Modem - A modem with a CPU (microprocessor) on board that uses the Hayes AT command set. This allows auto-answer & dial capability rather than manually dialing & answering.

- Digital Modem - Converts the RS-232 digital signals to digital signals more suitable for transmission. (also called a media adapter or transceiver)

- V.32 Modem - a milestone modem that uses a 2400 baud modem with 4 bit encoding. This results in a 9600 bps (bits per second) transfer rate. It brought the price of high speed modems below $5,000.

20. Explain the advantages and disadvantages of cable Modem

Ans:

The cable television companies have the high speed bandwidth to the homes. Cable modems use the existing cable TV line to provide the high speed bandwidth. It is an asymmetrical transfer rates with the upstream data transfer rate at 2 Mbps. The downstream data transfer rate is a maximum of 30 Mbps. Most users connect the cable modem to their 10 Mbps Ethernet NIC, and don't utilize the cable modems full bandwidth. Switching to a 100 Mbps Ethernet NIC would give them full bandwidth. Most cable companies use dynamic IP addressing: each time the user connects, they are assigned a new IP address. Most cable TV companies are placing high performance web proxy servers at the head end. These servers store the most commonly accessed web pages and files locally at the head end. The user's web browser first checks the proxy server to see if the file has been downloaded there. If it hasn't, then it goes out on the Internet to download it. The storing of the web pages and files on the local proxy server reduces the load on the communication servers (to the Internet), and gives the impression of extremely fast Internet access.

The cable modem is connected to the existing cable TV RG59 coax line, using a standard RF connector. The output of the cable modem is a 10BaseT or 100BaseT Ethernet connection to your NIC.

Cable Modem Advantages

- Fast data transfers, up to 30 Mbps if using a 100BaseT NIC

- Competitive pricing against competing technologies

- Easy to install - home prewired

Cable Modem Disadvantages

- The available bandwidth depends on the number of users on the local cable TV line segment.

- There is an asymmetrical transfer rate. Upstream is slower than downstream.

- There can be a bottleneck at the communication server at the head end.

21. Write Short note on CDMA

Ans:

CDMA means Code-Division Multiple Access, a digital cellular technology that uses spread-spectrum techniques. Unlike competing systems, such as GSM, that use TDMA, CDMA does not assign a specific frequency to each user. Instead, every channel uses the full available spectrum. Individual conversations are encoded with a pseudo-random digital sequence.

CDMA is a military technology first used during World War II by the English allies.

CDMA is  orginally known as IS-95, competes with GSM technology for dominance in the cellular world.There are now different variations, but the original CDMA is now known as cdmaOne.  The great attraction of CDMA technology from the beginning has been the promise of extraordinary capacity increase over narrowband multiple access wireless technologies.

22. What is Point-to-Point Topology

Ans:

Point-to-point topology is the simplest of the physical layouts of network devices. Point-to-point connections mean that two devices (nodes) have a single path for data to travel between them and there is nothing that breaks up that path.

The point-to-point topology can be seen as one of the basic building blocks of larger, more complicated topologies. All major topologies include point-to-point connections, even if there is no wire between two devices, but some other medium instead. Satellite transmissions are considered to be point-to-point communications. Similarly, laser transmissions can also be viewed in this manner. A variant on point-to-point connections is a multipoint topology in which a single cable may split into several segments in order to connect to several devices.  Point-to-point topology is not just limited to networking use. The direct connection of a PC to a printer follows a point-to-point topology.

23. Write shot notes on Hub

Ans:

Hubs can also be called either Multi port Repeaters or Concentrators. They are physical hardware devices. Hubs are used to provide a Physical Star Topology. At the center of the star is the Hub, with the network nodes located on the tips of the star. Hubs are a crucial element to all star topology LANs. Hubs serve as a central device through which data bound for a workstation travels. Hubs do not read any of the data passing through them and are not aware of their source or destination. Essentially, a hub simply receives incoming packets, possibly amplifies the electrical signal, and broadcasts these packets out to all devices on the network - including the one that originally sent the packet. The data may be distributed, amplified, regenerated, screened or cut off.  The hierarchical use of hubs removes the length limitation of the cables.

Hub joins multiple computers (or other network devices) together to form a single network segment. On this network segment, all computers can communicate directly with each other.

A hub includes a series of ports that each accept a network cable. Small hubs network four computers.

Hubs classify as Layer 1 devices in the OSI model.( physical layer)

Three different types of hubs exist:

- passive

- active

- intelligent

Passive hubs do not amplify the electrical signal of incoming packets before broadcasting them out to the network. Active hubs, on the other hand, do perform this amplification, as does a different type of dedicated network device called a repeater. They can also be treated as concentrator when referring to a passive hub and multiport repeater when referring to an active hub.

Intelligent hubs add extra features to an active hub that are of particular importance to businesses. An intelligent hub typically is stackable (built in such a way that multiple units can be placed one on top of the other to conserve space). It also typically includes remote management capabilities.

24. Write a note on synchronous voice grade modems.

Ans:

Synchronous modems operates in the audio domain, at rates up to 28800 bps in audio lines, used in telephones systems (using synchronous data). The usual modulation methods are the phase modulation and integrated phase and amplitude (at higher rates than 4800 bps).

Synchronous modems operate in the same manner asynchronous modems. However, synchronous modems operates at higher rates and since the requirements to transmit at these rates is increasing, most of the innovations are implemented for synchronous modems.

Synchronous data is accompanied by a clock signal. Synchronous data is almost always grouped in blocks, and it is the responsibility of the data source to assemble those blocks with framing codes and any extra bits needed for error detecting and/or correcting according to one of many different protocols (BISYNC, SDLC, HDLC, etc.). The data source and destination expect the modem to be transparent to this type of data, conversely, the modem can ignore the blocking of the data.

Voice-grade modems are used for unlimited destination, using a moderate to high data rate. These modems are expensive and their maintenance and tuning are sophisticated. Communication channels are leased lines and dial-up.

Voice-band telephone network is used for data transmission. A user-to-user connection may be either dedicated or dialed. The links in the connection are the same in the two cases, and the only difference for the user is that for some impairments (particularly attenuation and delay distortion), a dedicated (private or leased) line is guaranteed to meet certain specifications, whereas a dialed connection can only be described statistically.

25. Write short notes on GSM services

Ans:

GSM

Global System for Mobile Communications. The first European digital standard, developed to establish cellular compatibility throughout Europe. It's success has spread to all parts of the world and over 80 GSM networks are now operational. It operates at 900 MHz.Now  Global System for Mobile Communications is one of the leading digital cellular systems. GSM uses narrowband TDMA, which allows eight simultaneous calls on the same radio frequency. GSM was first introduced in 1991. As of the end of 1997, GSM service was available in more than 100 countries and has become the one of the popular standard in Europe and Asia.

GSM provide a lot of services some of them are Short Message Service which allows you to send and receive 126 character text messages. Ability to use same phone in a number of network-related countries. Allows data transmission and reception across GSM networks at speeds up to 9,600 bps currently. Forwarding of calls to another number. Place a call on Hold while you access another call.You can barr outgoing calls and incoming calls.

- Discuss various layers of OSI model.

Ans:

Application (Layer 7)

This layer supports application and end-user processes. Communication partners are identified, quality of service is identified, user authentication and privacy are considered, and any constraints on data syntax are identified. Everything at this layer is application-specific. This layer provides application services for file transfers, e-mail, and other network software services. Telnet and FTP are applications that exist entirely in the application level. Tiered application architectures are part of this layer.

Presentation (Layer 6)

This layer provides independence from differences in data representation (e.g., encryption) by translating from application to network format, and vice versa. The presentation layer works to transform data into the form that the application layer can accept. This layer formats and encrypts data to be sent across a network, providing freedom from compatibility problems. It is sometimes called the syntax layer

Session (Layer 5)

This layer establishes, manages and terminates connections between applications. The session layer sets up, coordinates, and terminates conversations, exchanges, and dialogues between the applications at each end. It deals with session and connection coordination.

Transport (Layer 4)

This layer provides transparent transfer of data between end systems, or hosts, and is responsible for end-to-end error recovery and flow control. It ensures complete data transfer.

Network (Layer 3)

This layer provides switching and routing technologies, creating logical paths, known as virtual circuits, for transmitting data from node to node. Routing and forwarding are functions of this layer, as well as addressing, internetworking, error handling, congestion control and packet sequencing

Data Link (Layer 2)

At this layer, data packets are encoded and decoded into bits. It furnishes transmission protocol knowledge and management and handles errors in the physical layer, flow control and frame synchronization. The data link layer is divided into two sublayers: The Media Access Control (MAC) layer and the Logical Link Control (LLC) layer. The MAC sublayer controls how a computer on the network gains access to the data and permission to transmit it. The LLC layer controls frame synchronization, flow control and error checking.

Physical (Layer 1)

This layer conveys the bit stream - electrical impulse, light or radio signal -- through the network at the electrical and mechanical level. It provides the hardware means of sending and receiving data on a carrier, including defining cables, cards and physical aspects. Fast Ethernet, RS232, and ATM are protocols with physical layer components.